STM V4.4:

Comments:

Currently based on BandsV2.7 (Fundamental Physics is based on V2.4)

No need to touch Column 1,8;

Science WGs fill columns 2-5;

SH fills columns 6,7;

Red highlight means – needs to be completed

Fundamental Physics (Raphael)

| 1. Science Goals | 2. Science Objectives | Scientific Measurement Requirements |  |  | Instrument |  | 8. Mission Functional Requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3. Model Parameters | 4. Physical Parameters | 5. Observables | 6. Functional Requirements | 7. Projected Performance |  |
| Explore how the universe began (Inflation) | 1. Probe the physics of the big bang by detecting the energy scale at which inflation occurred if it is above 4x1015 GeV, or place an upper limit if it is below (see Figure TBD) | Tensor-to-scalar ratio r: (r) < 5x10-5 at r = 0 r<10-4 at 95% CL (1) | CMB polarization B-mode power spectrum for modes 2<l<300 to cosmic variance limit, and CMB lensing power spectrum for modes 2<l<1000 to cosmic variance limit | Linear polarization at frequencies 60<<300 GHz over the entire sky | 21 frequency bands between 20 and 800 GHz; fractional bandwidth of 25% (2) FWHM of 6.2’ at 155 GHz and remaining single mode resolution to higher frequencies (3) Combined instrument noise weight of 0.7 uK*arcmin | 21 frequency bands between 20 and 800 GHz; fractional bandwidth of 25% 7.6’ resolution at 150 GHz Combined instrument weight of 0.7 uK*arcmin | Sun-Earth L2 halo orbit Mission life 4 yr Full sky survey Survey efficiency 95% Downlink 150 Mbits/hour; 6 hour/day TBD Spinning/precessing TBD Pointing accuracy TBD Pointing stability TBD Thermal TBD Sun avoidance TBD Launch vehicle compatibility |
|  | 2. Probe the physics of the big bang by excluding classes of potentials as the driving force of inflation (see Figure TBD) | Spectral index ns and its derivative (ns) < 0.0015 (nrun) < 0.002 | CMB polarization B-mode power spectrum for modes 2<l<1000 to cosmic variance limit | Intensity and linear polarization at frequencies 60<<400 GHz over the entire sky |  |  |  |

- The values includes delensing and ‘simple’ foreground subtraction across the 21 frequency bands

- Broad frequency for foreground separation

- Resolution required for delensing and foreground characterization

Fundamental Physics (Raphael) + reionization (Nick)

| 1. Science Goals | 2. Science Objectives | Scientific Measurement Requirements |  |  | Instrument |  | 8. Mission Functional Requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3. Model Parameters | 4. Physical Parameters | 5. Observables | 6. Functional Requirements | 7. Projected Performance |  |
| Discover how the universe works (Neutrino Mass and Neff) | 3. Determine the sum of neutrino masses, and distinguish between inverted and normal neutrino mass hierarchies (See Figure TBD) | Sum of Neutrino masses: (m) < 15 meV (4) | CMB polarization B-mode power spectrum for modes 2<l<4000 to cosmic variance limit; CMB intensity maps (to give Compton Y map from which we extract clusters) | Intensity and linear polarization at frequencies 60<<400 GHz over the entire sky | As above | As above | As above |
|  | 4. Detect departures from or tightly constrain the thermal history of the universe | Number of effective relativistic degrees of freedom (Neff) < 0.03 | CMB temperature and E-mode polarization power spectra 2<l<4000 to cosmic variance limit | Intensity and linear polarization at frequencies 60 <<300 GHz over the entire sky |  |  |  |
| Explore how the universe evolved (reionization) | 5. Distinguish between models that describe the formation of the earliest stars in the universe. (see Figure TBD) | Depth to reionization : () < 0.002 | CMB polarization E-mode power spectrum for modes 2<l<20 to cosmic variance limit; T power spectrum and Compton Y maps. | Intensity and linear polarization at frequencies 60 <<300 GHz over the entire sky (role of intensity maps at high \ell to be clarified) | Frequency bands as above Resolution of 1 deg; (role of intensity maps at high \ell to be clarified) Combined instrument weight of 0.7 uK*arcmin (Perhaps can survive with less sensitivity) |  |  |

- Using the PICO lensing power spectrum and tau, and BAO from DESI; or independently using our cluster counts and LSST data

Extragalactic Science (Nick)

| 1. Science Goals | 2. Science Objectives | Scientific Measurement Requirements |  |  | Instrument |  | 8. Mission Functional Requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3. Model Parameters | 4. Physical Parameters | 5. Observables | 6. Functional Requirements | 7. Projected Performance |  |
| Explore how the universe evolved (galaxy formation, & feedback) | 6. Determine the role of energy injection due to feedback processes on galaxy formation and evolution | The baryon density and electron pressure radial profile of galaxy halos of mass M>1013.5 Msun/h | All sky CMB temperature and Compton Y maps | Temperature at frequencies 60<<400 GHz over the entire sky | Beam size of 2.5 arcmin at 385 GHz and increasing with single mode dependence to lower frequencies | As above | As above |

Galactic Science (Laura + Dave)

| 1. Science Goals | 2. Science Objectives | Scientific Measurement Requirements |  |  | Instrument |  | 8. Mission Functional Requirements |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3. Model Parameters | 4. Physical Parameters | 5. Observables | 6. Functional Requirements | 7. Projected Performance |  |
| Explore how the universe evolved (magnetic fields) | 7. Determine if magnetic fields are the dominant cause of low star formation efficiency in our Galaxy. | B - Magnetic Field strength as a function of spatial scale and density; a - Alfven Mach number (ratio of turbulent energy to magnetic energy); p, Plasma beta (ratio of thermal energy to magnetic energy). | The turbulence power spectrum on scales ?? (from cores to diffuse cloud envelopes); Fractional polarization level; Correlations of fractional polarization and direction with hydrogen column density and temperature. | Linear polarization at = 799 GHz for galactic latitudes -20 <b<20 (to obtain maps of thousands of molecular clouds with <1pc resolution and <0.05 pc for the 10 nearest MCs.) | Frequency band at 799 GHz; Angular resolution of 1 arcmin. Sensitivity of 27.4 KJy/Sr | Frequency band at 799 GHz; Angular resolution of 1 arcmin. Sensitivity of 27.4 KJy/Sr | Same as above |
|  | 8. Determine whether the interstellar medium of our galaxy is unique by comparing the ratio of energy in magnetic field to turbulence to that in nearby galaxies. | B - Magnetic Field strength; a - Alfven Mach number | Magnetic field maps of nearby external galaxies (how many?); Levels of fractional polarization | Linear polarization at = 799 GHz to obtain maps of a statistically-significant set of external galaxies. |  |  |  |
|  | 9. Determine whether radiative torque is responsible for the alignment of dust grains with magnetic fields | Polarization spectra (fractional polarization as a function of wavelength) | This needs to be something like Tau_radiative (a parameter that quantifies radiative torque) and A_grain (a parameter that quantifies grain alignment). | Linear polarization maps in several frequency bands between 150 and 799 GHz for regions with high and low radiative flux | 10 frequency bands between 150 and 799 GHz; To be completed |  |  |
|  | 10. Determine the variations in temperature and spectral index of polarized dust emission. | To be filled in | To be filled in | To be filled in |  |  |  |
| Explore how the universe evolved (magnetic fields) | 11. (needs more cooking)Is the level of magnetized turbulence consistent with observations of Cosmic Ray energy spectra? (why?) | B - Magnetic Field strength as a function of spatial scale; a - Alfven Mach number | Magnetic field maps of the diffuse ISM. The turbulent power spectrum in regions of low intensity. | Linear polarization at frequencies > 300GHz over the entire sky, with <0.1 pc resolution for the edge of the local bubble (d~100pc). | Sensitivity: A_v <0.1(need to convert to Jy/sr), < 4 arcmin resolution |  |  |

Legacy Science (Nick, Gianfranco)

| Catalog | Impact | Science |
| --- | --- | --- |
| 1. proto-clusters | Discover thousands (how many) of new proto-clusters selected homogeneously over the entire sky and in redshift. Currently there are less than ?? identified (compared to heterogeneous samples of a few) | Explore how the universe evolved: Probe the earliest observable galaxy clusters to determine the initial stages of their formation and evolution |
| 2. Lensed Point sources | 3,000 highly magnified dusty sources. Currently there are ?? sources found by the Planck satellite | Explore how the universe evolved: Learn about dark matter sub-structure in the lensing galaxies; probe star formation history in high-z dust enshrouded galaxies, a population in which star formation history can not be probed in any other way |
| 3. High-z galaxy clusters | Hundreds (200 or 800?) of virialized (opaque to the non-specialist, use a different word) clusters at z >2 | galaxy formation in dense environments at the peak of star formation for most cluster galaxies, ICM,… |
| 4. Polarized Point Sources | Detection of thousands(how many) of radio sources and dusty galaxies in polarization | Explore how the universe works: Measure the polarization properties of dusty and radio sources, and determine of their impact on CMB polarization science. |

Top of Form

Bottom of Form

Shaul’s additional comments:

Please review the science column

Line #3 needs a thorough explanation